# Bias Voltage DAC Specifications

Draft v0 | October 4, 2026 | For mentor discussion

Proposed application: Low-power DC bias for high-impedance quantum-dot gates. All targets below are provisional and have not been demonstrated.

| Parameter | Draft target | Verification or decision |
| --- | --- | --- |
| Channels | 1 channel first; 8-channel extension | Confirm project scope |
| Reference span | 1 V; absolute voltage and polarity TBD | Loaded output-range test |
| Resolution | 12-bit nominal; 244 µV/LSB | Full-code transfer curve |
| Linearity | INL and DNL within ±0.5 LSB; calibration method TBD | Raw and corrected INL/DNL; monotonicity check |
| Update and settling | 1 kupdate/s; settle within 100 µs to ±0.25 LSB | Worst-case code transitions at specified load |
| Hold stability | Droop ≤0.25 LSB (61 µV) over 1 ms | Hold transient across codes; measure turn-off step separately |
| Output load | High impedance; capacitance and leakage TBD | Load and leakage sweeps |
| Power | ≤5 µW/channel, including allocated reference, control and buffer power | Average energy from all supply, reference and control ports |
| Noise | RMS limit and bandwidth TBD | Noise analysis with defined bandwidth |
| Temperature | 27 °C baseline; 4.2–6 K validation when models or measurements exist | Validated cryogenic model or measurement; mK deferred |

Development path: 3-bit functional prototype → 8-bit feasibility → 12-bit application target.

Mentor decisions: Confirm the actual load/application, required accuracy and calibration, and cryogenic validation scope.

Open budget: Static error, settling, droop, noise and switching disturbance must be combined before claiming total accuracy.

Application anchor: Vliex et al., IEEE SSC-L (2020), DOI 10.1109/LSSC.2020.3011576. Other targets are engineering proposals; detailed rationale is in the companion specification draft.