# Bias Voltage DAC Design Specifications

Version 0 revision 2 | Full text update | October 4, 2026

Starting in Week 7, use these specifications to select circuit parameters and plan simulations unless a later project decision changes them. The working points below are provisional internal design targets selected from the discussion ranges; they are not mentor-approved system requirements or achieved performance.

# Working design baseline

| Item | Default for design work |
| --- | --- |
| Application | Slow programmable DC bias into a high-impedance node; confirm whether the load is a quantum-dot gate or the collaborator analog/Delta-Sigma interface. |
| Architecture | Continue CDAC + TG. Validate the 3-bit prototype, assess 8-bit scaling, and design toward a 12-bit application point. Keep MIM for the baseline. |
| P1 Accuracy | 12-bit / 1 V gives 244.14 uV/LSB. Working max ¦INL¦ <=1 LSB and ¦DNL¦ <=0.5 LSB; strictly monotonic. |
| P2 Power | <=5 uW per allocated channel at 1 kupdate/s, including reference, control and any output buffer. |
| P3 Temperature | Target envelope 4.2-300 K; currently only a nominal 27 C simulation baseline. Low-temperature verification requires valid models or measurements. |
| Secondary screens | RT settling <=10 us to +/-61.0 uV; drift <=61.0 uV/1 ms; noise <=61.0 uVrms over 1 Hz-10 kHz; valid-window ripple <=61 uVpp. |

# Current implementation and open constraints

The 3-bit TSMC65 array has six TG reference switches and eight nominal 110.659 fF MIM units. Reset is still ideal; references and Verilog-A control are ideal/behavioral. The TT, 27 C, 1 V, no-added-load scan reached 0 to 874.9987 mV with an 8.4 uV maximum displayed marker error. This is a functional baseline, not proof of 12-bit linearity or cryogenic performance.

Use native LSB for INL/DNL at each implemented bit count. The 61.0 uV timing/noise/drift screens come from the future 12-bit budget and may be used to expose limits in the 3-bit testbench without calling it a 12-bit DAC. The 5 ns dead time and 1 ns control edges are test settings, not rated update speed.

A 10 uV step over 1 V needs at least 17 bits; alternatively, 12 bits cover at most a 40.96 mV fine window at that step. R24J therefore motivates an application decision, not an automatic change to an 18-bit R-2R architecture. Actual load, output range, reference allocation and total error budget remain open.

# Specification ranges and working points

P1-P3 are primary; S is secondary. A ceiling range is the set of candidate limits, not a range of acceptable errors. Literature evidence and per-row rationale are detailed in the companion Markdown. The working point allows progress before the final system contract is agreed.

| Metric | Priority | Range and working condition |
| --- | --- | --- |
| Resolution and DC accuracy | P1 | CDAC scope 11-13 bits; work at 12 bits / 1 V. Max ¦INL¦ ceiling range 0.5-2 LSB; work at <=1 LSB. ¦DNL¦ <=0.5 LSB goal; DNL > -1 required. |
| Power per channel | P2 | Budget range 3-10 uW/ch at 1-4 kupdate/s; working ceiling 5 uW/ch at 1 kupdate/s. Include allocated reference, control and buffer power. |
| Temperature robustness | P3 | Intended envelope 4.2-300 K; validate only supported model/measurement points. Same accuracy/power limits; 1-2x RT settling ceiling range, working cap 2x (20 us). |
| Reference and supply | S | Working bench VDD=1 V; Vu/Vl=1/0 V. Supply exploration 0.9-1.1 V only inside PDK ratings; output polarity/span pending interface agreement. |
| Settling and update | S | Range 1-4 kupdate/s and 10-100 us settling. Work at 1 kupdate/s and <=10 us to +/-0.25 LSB12 (+/-61.0 uV) at RT. |
| Hold drift and refresh | S | Drift allowance 0.1-0.25 LSB12 over 1 ms; work at <=61.0 uV/1 ms. Explore refresh 0.1 Hz-4 kHz; do not assume low-rate operation passes. |
| Noise and reference stability | S | Range 0.1-0.25 LSB12 rms; working ceiling 61.0 uVrms over 1 Hz-10 kHz. Also report 1 Hz-50 kHz for comparison. Reference budget not yet allocated. |
| Load and switching disturbance | S | Load probes: added C=0,10 fF,100 fF,1 pF,10 pF; leakage=10 fA-10 pA. During valid hold window, explore ripple <=24-61 uVpp; work at <=61 uVpp. |
| Area and capacitor choice | S | No area acceptance ceiling yet. Keep MIM baseline; compare one legal MOM option at equal capacitance, with matching and parasitics. |
| FoM and effective resolution | S | At 1 kHz, budget 3-10 nJ/ch/update; working ceiling 5 nJ/ch/update from 5 uW. Report idle power separately; no ENOB-normalized target. |

The targets do not constitute a closed total-error budget. Static error, settling residual, drift, reference error and noise must be allocated together. Glitch during blanking and voltage during the valid hold window are separate metrics.

# Core literature comparison

| Study | Architecture | Measured performance | Power and size | Temperature interpretation |
| --- | --- | --- | --- | --- |
| R01 Vliex 2020 | 65 nm; coarse/fine CDAC with 8 S&H outputs | 6 K; 13-bit code; 1 V span | 3.18 uW/ch with clock buffer; 3.9 kHz refresh | No complete like-for-like RT/cold DAC metric set extracted; do not infer degradation from the op-amp measurements. |
| R09 Miki 2022 | 40 nm; 11-bit split CDAC; 6 fF MOM | 300 K and 8 K; INL about +/-2 LSB; DNL -2 to +0.5 LSB after calibration | 5.8 uW at 3.9 kHz; 0.008 mm2 core | Similar plotted RT/cold linearity; retention improves on cooling. DNL does not demonstrate monotonicity. |
| R04 Enthoven 2022 | 22 nm FinFET; integrating DAC; >480 pF measured load | 300 to 4.2 K: step 68.3 to 57.1 uV; INL 12.6 to 36.5 LSB; DNL 0.6 to 0.8 LSB | 138 to 157 uW; 62 kHz clock; 0.076 mm2 core | Cold/RT ratios: power 1.14, step 0.84, INL 2.90, DNL 1.33. Noise 192 to 188 uVrms. Do not treat small steps as equal accuracy. |

R01, R09 and R04 are independent designs. R02 and R03 are follow-ups of R01; R04/R05 and the ISSCC/JSSC Xiling records also represent shared research lines. Do not count these extensions as independent benchmarks.

# Energy and power boundaries

| Study | Derived allocation | Boundary |
| --- | --- | --- |
| R01 | 0.815 nJ/ch/refresh | 3.18 uW/ch / 3.9 kHz; clock buffer included, external reference generation excluded. |
| R09 | 1.49 nJ/DAC/update | 5.8 uW / 3.9 kHz; stated DAC power only. |
| R03 | 2.5 uW/output; 0.642 nJ/output/refresh | 20 uW / 8 / 3894.1 Hz. Allocation across eight outputs, not an independently measured per-output power. |
| R04 | 2.53 nJ/integration cycle | 157 uW / 62 kHz; not a complete settled channel update. |
| R15 and R24J | Static power comparison only | R15 7.3 uW static; R24J about 60 uW dual-DAC DC mode. Do not divide by estimated/max clock speed to claim measured update energy. |

There is no directly comparable universal FoM across these architectures. Report idle power and the event definition; nominal resolution is not measured ENOB. The 5 nJ working allocation at 1 kHz follows the 5 uW power ceiling.

# Capacitor choice

Keep MIM until a legal TSMC65 MOM option is compared at equal capacitance. Check density, minimum size, metal usage, matching, voltage coefficient and terminal parasitics with layout/PEX. R09 uses 6 fF MOM; R10 measures 191 to 197 fF from RT to 4 K in a different 40 nm process. Neither establishes that MOM is better for this project.

# Evidence added from the three supplied papers

| Study | Circuit and scope | Verified results | Important limits |
| --- | --- | --- | --- |
| R03 SiGe 2023 | 65 nm CDAC; 13 bits, 0-1 V; 8 S&H outputs. Same research line as R01/R02. | Measured: about 20 uW DAC, 40 uW whole IC; 0.96 uV/s drift with clock off; 275 uVpp ripple at 3.8941 kHz. | MC 44 mK, holder 180 mK, electrons about 400 mK. 64.5 pW/ch and 0.1 Hz scaling are extrapolated. |
| R15 Zurita 2020 | 28 nm FDSOI; 8-bit current steering; 6.6 mV differential span, about 26 uV step. | At 4.2 K: 7.3 uW static including digital; max DNL 0.64 LSB, max ¦INL¦ 2.96 LSB. No calibration. | RT max DNL/INL: 0.11/0.63 LSB. 350 ns measured RT rise at 35.2 pF; 100 MS/s is estimated, not measured at cryo. |
| R24J Xiling 2026 | 65 nm R-2R; dual 18-bit DACs; 1.2 V span, 4.58 uV step; RNC + OEM calibration. | At 4 K after calibration: ¦INL¦, ¦DNL¦ <=0.8 LSB. Chip DC power 61.6/52.1/59.9 uW at 300 K/4 K/60 mK. | Reusing 4 K calibration at 10 K: DNL +6/-2, INL +5/-8 LSB. 60 mK is environment; SET electron estimate about 900 mK. |

# Consequences for this design

Retention and ripple [R03]: the 0.96 uV/s drift was reconstructed from a SET over 13 minutes with the clock stopped. The 275 uVpp ripple includes refresh and inter-channel coupling. The extrapolated 64.5 pW/ch is not measured eight-channel power. Test clock-off hold, active refresh and cross-coupling separately.

Noise [R24J]: Table I gives 8.6, 2.6 and 3.8 uVrms over 1 Hz-50 kHz at 300 K, 4 K and 60 mK. Measurements use zero DAC code to suppress supply influence; 4 K ground noise is deembedded, while 60 mK retains residual refrigerator noise. The 4.1 nV/sqrt(Hz) value is at 10 kHz, not integrated noise. Our nonzero-code/reference-noise tests remain necessary.

Calibration and temperature [R24J]: uncalibrated 4 K DNL is +5/-10 LSB and INL +25/-45 LSB; OEM plus RNC yields +/-0.8 LSB. Reusing those coefficients at 10 K degrades both metrics. Report raw, fixed-calibration and recalibrated performance separately. These resistor results cannot be copied into TSMC65 capacitor/mismatch models.

Scope and speed [R15]: 26 uV steps come from an approximately 6.6 mV fine range, not from high resolution over 1 V. Its 100 MS/s claim is extrapolated from an RT load-dependent rising-time test; no cryogenic dynamic test was possible. Keep update rate, internal clock, RC time constant and precision settling distinct.

All three requested full texts are now available. R15 is supplied as an author manuscript despite its ESSCIRC filename; its title and content match the requested SSC-L paper. R24J is the published 2026 journal extension, not an additional independent ISSCC design.

# Verification and Week 7 design sequence

| Check | Simulation | Evidence and working limit |
| --- | --- | --- |
| Static transfer | Ascending/descending all-code sweeps; fixed sample delay; repeated initial states. Use native LSB for each implemented resolution. | Report raw error, endpoint INL/DNL, offset/gain, minimum step and hysteresis. Working eventual limits: ¦INL¦<=1, ¦DNL¦<=0.5 LSB; monotonic. |
| Switch and transient | 011<->100 and 000<->111 on 3-bit core; sweep edge, dead time, W and output voltage. Confirm timestep convergence. | <=10 us request-to-settle at RT within 61.0 uV of its own final level; also report final target error. Peak, area and valid-window ripple separately. |
| Retention and noise | Hold 1 ms; sweep leakage/load and codes. Separate clock-off drift, refresh ripple, and random sampled/reference noise. | Working limits: drift <=61.0 uV/1 ms; ripple <=61 uVpp in valid window; rms noise <=61.0 uV over 1 Hz-10 kHz. Not a combined accuracy claim. |
| Power and load | Integrate energy at every physical supply/reference/control port; separate idle/update; sweep Cload and reference impedance. | Behavioral-driver output energy is load-delivered energy, not a model of decoder internal power. Recheck output range under load. |
| Robustness | Supported PVT, MOS/cap mismatch and PEX. Low-temperature cases require validated models or measurements; save calibration per temperature. | Compare uncalibrated, fixed-calibration and recalibrated results. Same accuracy/power caps, RT settling <=10 us and cold <=20 us. Record actual temperature location. |

# Next design steps

| No. | Task | Decision enabled |
| --- | --- | --- |
| 1 | Save the 3-bit baseline and complete reset TG implementation | Compare against the ideal-reset case; preserve the original regression. This closes the remaining ideal switch before sizing decisions. |
| 2 | Create spec-linked ADE measurements | Ascending/descending codes, major-carry/full-scale transitions; raw/static errors, settling, ripple and timestep convergence. |
| 3 | Determine load and error sensitivities | Run the listed C/leakage probes; identify gain loss and hold limits. Agree the real interface with the mentor before qualifying a loaded design. |
| 4 | Choose TG and capacitor sizes from the failed limits | Use settling, injection, noise and power together. Keep MIM until equal-C MOM evidence warrants a change; assess 8-bit scaling before 12 bits. |

Size from constraints: Req <= t_settle / [Ceff ln(deltaV/error)] is a first-order guide, not a fixed Ron target. For one floating-top-plate branch, Ceff = Cb(Ctotal-Cb)/Ctotal; the present MSB gives about 221 fF. Multi-bit transitions require the complete circuit.

For hold, deltaV = Ileak Thold / Chold: 1 pF and 61 uV/1 ms imply about 61 fA. Added load also changes charge-sharing gain: a 1 pF load on the 0.885 pF array gives a simple gain factor about 0.469. Agree the actual load before claiming output accuracy.

Before changing a device, name the spec it addresses. After the change, rerun that measurement and the baseline code regression. Record the condition, target, result and pass/fail or unverified status. Final system sign-off still needs the mentor to confirm application, load/span, temperature/model support, calibration and power boundary.

# References and local full text status

Reference IDs match the 24-record review; R24J identifies the journal extension of R24. The three user-supplied PDFs have been read and are stored under papers. There are no outstanding downloads from the previous three-paper request.

[R01] P. Vliex et al. Bias Voltage DAC Operating at Cryogenic Temperatures for Solid-State Qubit Applications. IEEE Solid-State Circuits Letters, vol. 3, pp. 218-221, 2020. DOI: 10.1109/LSSC.2020.3011576. Evidence: Sections IV and V; Tables I and II.

[R09] T. Miki, R. Takahashi, and M. Nagata. An 11-bit 0.008 mm2 charge-redistribution digital-to-analog converter operating at cryogenic temperature for large-scale qubit arrays. IEICE Electronics Express, vol. 19, no. 8, article 20220099, 2022. DOI: 10.1587/elex.19.20220099. Evidence: Section 4; Figures 7-9; Table I.

[R04] L. Enthoven, J. van Staveren, J. Gong, M. Babaie, and F. Sebastiano. A 3V 15b 157uW Cryo-CMOS DAC for Multiplexed Spin-Qubit Biasing. IEEE Symposium on VLSI Technology and Circuits, pp. 228-229, 2022. DOI: 10.1109/VLSITechnologyandCir46769.2022.9830309. Evidence: Page 229, Table 1 and Figure 6.

[R02] R. Otten et al. Qubit Bias using a CMOS DAC at mK Temperatures. IEEE ICECS, 2022. DOI: 10.1109/ICECS202256217.2022.9971043. Evidence: Sections II and III; Table I.

[R10] B. Patra et al. Characterization and Analysis of On-Chip Microwave Passive Components at Cryogenic Temperatures. IEEE Journal of the Electron Devices Society, vol. 8, pp. 448-456, 2020. DOI: 10.1109/JEDS.2020.2986722. Evidence: Section III and Table 1.

[R11] P. A. t Hart et al. Characterization and Modeling of Mismatch in Cryo-CMOS. IEEE Journal of the Electron Devices Society, vol. 8, pp. 263-273, 2020. DOI: 10.1109/JEDS.2020.2976546. Evidence: Sections IV and V.

[R18] J. van Staveren et al. Cryo-CMOS Voltage References for the Ultrawide Temperature Range From 300 K Down to 4.2 K. IEEE Journal of Solid-State Circuits, vol. 59, no. 9, pp. 2884-2894, 2024. DOI: 10.1109/JSSC.2024.3378768. Evidence: Sections IV and V.

[R03] L. Schreckenberg et al. SiGe Qubit Biasing with a Cryogenic CMOS DAC at mK Temperature. IEEE ESSCIRC, pp. 161-164, 2023. DOI: 10.1109/ESSCIRC59616.2023.10268801. Evidence: Sections II-III; Table I; Figure 3 (PDF pp. 2-3).

[R15] M. E. P. V. Zurita et al. Cryogenic Current Steering DAC With Mitigated Variability. IEEE Solid-State Circuits Letters, vol. 3, pp. 254-257, 2020. DOI: 10.1109/LSSC.2020.3013443. Evidence: Sections III-V; Figures 7-9; Table I (author manuscript pp. 3-4).

[R24J] Y. Li, Y. Zhang, H. Lin, and C. Wang. Xiling: Cryo-CMOS Manipulator Using Dual 18-bit R-2R DACs for Single-Electron Transistor at 60 mK. IEEE Journal of Solid-State Circuits, vol. 61, no. 8, pp. 4193-4205, 2026. DOI: 10.1109/JSSC.2026.3671795. Evidence: Section V; Figures 20-25; Table I (PDF pp. 9-12).

# Revision and application status

Revision 2 adds full-text evidence for R03, R15 and R24J and establishes the Week 7 working baseline. Discussion ranges remain visible as requested by the mentor guide. Some timing, noise, load and error allocations are explicit engineering assumptions, not yet application-approved requirements. From Week 7, we will use these targets to guide design and revise them as the application and mentor feedback become clear.